HUMMINGBIRD HOUSE

- Pre drill the 7”x2”x2” for the house opening and perch

House opening-7/8”- 1 1/2 “from the top end

Perch- 1/8”-1” below the 7/8” hole

- Mount the blank between centers with the house opening at the head stock end.

- Turn the blank round to 2” diameter

- Turn a tenon on the tail stock end

- Remount the piece in your chuck

- Remove the live center from the tailstock

- Mount a Jacobs chuck with a 1 3/8” forstner bit in the tailstock

- Keep the lathe speed low, approx. 200 RPM

- Drill into the blank approx. 1” past the perch hole

- Remove the bit and chuck from tailstock

- If you have a Oneway live center with a cone, place it in the tailstock and slide it into the hole just finished

- If you don’t have the cone you can make one to go over your live center.  Make sure it’s a snug fit and it revolves

- Turn down the piece to 1 3/4” leaving 3/16” wall

- Remove your piece. If you have spigot jaws mount them on your chuck.  If not make jamb chuck with tenon for your chuck and a 1 3/8 “tenon to slide the house over. Use caution here when you mount the house so as not so crack the thin wall

- Bring up the tailstock with your live center for support

- Finish turning the bottom part of your piece, remember how deep you drilled with the forstner bit!!!   Begin your tapering after this

- Finish the piece.  Remove the tailstock when 99% turned.  Remembering to turn the hole (left over from live center) away

LID / TOP

- Start with a blank- 2 1/2 “x 2 1/2” x 2”

2.   Make center points on both sides

3.  Draw a 2” circle

4.  Cut on the ban saw or hand saw to remove the corners

5.  Mount between centers and turn round

6.  Make a tenon on one end to fit your chuck

7.  Mount the chuck on the lathe and mount the tenon in the chuck.  Make a 1 3/8”x 1/4“deep tenon to fit the bird house.   This is made at the chuck end.

8.  Turn the top.  Don’t worry about the live center hole as you will be installing a hook there to hang up the bird house.  Top can be threaded or glued.

PERCH

2 1/2” x 3/4"x 3/4 “blank

1.  Mount between centers and turn round

2.  Perch has a 1/8” x 3/16” long end to fit in the bird house

3.   Rest of the per should be approx. 1/4” diameter, with a slightly larger rounded end

4.  Glue the perch